Study of the Constituents of Different Alloys

INDEX

- Aim

- Introduction

- Materials Required

- Theory

- Procedure

- Experiment 1: Analysis of Brass

- Experiment 2: Analysis of Steel

- Observations & Calculations

- Results

- Diagrams / Microstructure Images

- Precautions

- Conclusion

- Future Scope

- Bibliography

1. AIM

To study the constituents of different alloys through chemical, physical, and microstructural analysis and compare their properties.

2. INTRODUCTION

An alloy is a metallic substance composed of two or more elements, at least one of which is a metal. Alloys are engineered to achieve enhanced strength, corrosion resistance, ductility, thermal conductivity, and other mechanical properties.
This project focuses on determining the constituents of two commonly used alloys—Brass and Steel—using analytical methods.

Alloys are mixtures of two or more elements, where at least one is a metal. They are created to enhance specific properties such as strength, hardness, durability, corrosion resistance, and conductivity, which pure metals often lack. Each alloy has a unique composition, and its behavior depends directly on the type and proportion of its constituent elements.

Studying the constituents of different alloys helps in identifying the metals present, understanding their chemical and physical characteristics, and relating these properties to real-world applications. This experiment focuses on analyzing various alloy samples using physical tests, chemical reactions, and basic analytical methods to determine their composition and identify the major elements present.

2. Materials Required

- Alloy samples (labelled A, B, C — e.g., brass, bronze, steel, nichrome or student-supplied unknowns)

- Analytical balance (±0.01 g)

- Beakers (50 mL, 100 mL)

- Conical flask (100–250 mL)

- Measuring cylinder (10–50 mL)

- Glass rod

- Filter paper and funnel

- Hot plate or Bunsen burner with tripod and wire gauze

- Dilute hydrochloric acid (dilute HCl, ~1–2 M)

- Dilute nitric acid (only if required and under supervision) — optional and only for trained labs

- Sodium hydroxide solution (NaOH, dilute)

- Distilled water

- Magnet

- Test tubes and rack

- Pipettes (dropper and volumetric)

- Pycnometer or density-measurement setup (overflow method) and water bath

- Watch glass and drying oven (or hot plate for drying)

- Safety equipment: lab coat, goggles, gloves, fume hood or well-ventilated area

Note: Always use acids and bases with appropriate safety measures and supervision. Replace strong oxidizers with safer alternatives where possible.

- 4. THEORY

- 4.1 Definition of an Alloy

- An alloy is formed by combining two or more elements where the resulting material has metallic properties and improved characteristics.

- 4.2 Types of Alloys

- Substitutional Alloys (e.g., Brass: Cu + Zn)

- Interstitial Alloys (e.g., Steel: Fe + C)

- Complex Alloys (e.g., Stainless steel)

4.3 Importance of Alloy Constituents

Each constituent contributes specific properties:

- Copper → ductility, electrical conductivity

- Zinc → strength, corrosion resistance

- Iron → hardness

- Carbon → strength and brittleness

- Chromium & Nickel → corrosion resistance (in stainless steel)

4. Procedure — Overview (applies to all samples)

- Label each alloy sample (A, B, C…). Record appearance, color, lustre, and any stamping/markings.

- Measure and record mass using an analytical balance (m_initial).

- Perform physical tests:

  - Magnet test: Check if attracted to a magnet — indicates presence of ferromagnetic elements (Fe, Ni, Co).

  - Spark/Grind test: (If allowed) grind a small piece against a grinding wheel and observe spark pattern — steel vs stainless.

  - Density measurement: Use a pycnometer or water displacement to estimate density.

- Perform simple chemical tests on a small piece or shavings:

  - Reaction with dilute HCl: Observe whether effervescence (H₂ evolution) occurs — indicates reactive metals like Zn, Mg, Al (but aluminium often passivates).

  - NaOH test: Add a few drops of NaOH

5. Experiments

Experiment 1 — Qualitative Identification of Alloys (Physical & Simple Chemical Tests)

Objective: Use physical and safe chemical tests to identify likely constituents of unknown alloy samples.

Steps:

- Visual & tactile inspection: Note color (yellowish for brass, reddish for copper-rich, silvery for steel/nichrome), grain, and surface finish.

- Magnet test: Bring a magnet near the sample. If strong attraction → presence of iron or nickel.

- Density test (quick method): Measure mass in air (m_air). Suspend sample in water and measure apparent mass in water (m_water_apparent) using either suspension on balance or via pycnometer. Density = m_air / (m_air − m_water_apparent).

- Reaction with dilute HCl (small shaving): Place a small shaving in 10 mL dilute HCl in test tube. Observe

  - Effervescence — indicates a reactive metal component such as Zn or Mg (for brass expect bubbling due to Zn).

  - No reaction — suggests less reactive metals (Cu, Au, Ag, Sn).

- NaOH test (on solution if dissolved): Add a few drops of NaOH to the acid solution (after neutralization) or to soluble metal salt solutions to test for amphoteric metals (Al, Zn form soluble complexes with excess NaOH).

Recording: Fill the qualitative observation table for each alloy.

Experiment 2 — Quantitative Determination of Composition: Brass (Cu–Zn) by Selective Dissolution

Objective: Determine percentage composition (mass%) of copper and zinc in a brass sample by dissolving zinc in dilute HCl and weighing the remaining copper.

Rationale: Zinc reacts with dilute HCl producing hydrogen gas and soluble ZnCl₂. Copper does not react with dilute HCl and will remain as a solid residue.

Steps:

- Weigh sample: Accurately weigh a small clean piece of brass sample (mass m_initial). Record.

- Dissolution: Place the alloy in a beaker or conical flask and add enough dilute HCl (1–2 M) to cover the sample. Set up under a fume hood or well-ventilated area. Gently warm (optional) until bubbling stops and no more reaction is observed. This dissolves zinc (Zn) leaving metallic copper and any insoluble impurities.

- Filter: Filter the mixture to collect the undissolved residue (primarily copper). Wash residue with distilled water several times to remove soluble Zn salts.

- Dry and weigh residue: Dry the residue in an oven or on a hot plate (use watch glass) until constant mass is reached. Cool in a desiccator and weigh (m_residue = mass of Cu + any insolubles).

- Calculations: Determine mass of zinc dissolved: m_Zn = m_initial − m_residue. Calculate percentage composition:

  - %Cu = (m_residue / m_initial) × 100

  - %Zn = (m_Zn / m_initial) × 100

Notes & Limitations:

- This method assumes only Zn dissolves and Cu remains inert to the reagent. If other alloying elements (e.g., Pb, Sn) are present, they may also remain or partially react — affecting accuracy.

- If alloy contains more than two components or reactive impurities, more advanced methods (AAS, ICP, XRF) are preferred.

6. Observation Table (sample format)

| Sample | Mass (g) | Appearance | Magnetism | Density (g·cm⁻³) | Reaction with HCl | Residue mass after HCl (g) | %Major metal (example) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A | 2.500 | Yellowish | No | 8.40 | Vigorous effervescence | 1.575 | %Cu = 63.0% |
| B | 2.000 | Silvery | Yes | 7.85 | No reaction | — | Fe present (magnetic) |

Fill this table with your measured values and observations for each sample.

7. Sample Calculation (for brass example)

Given: m_initial = 2.500 g (brass sample). After treatment with dilute HCl and drying, residue mass m_residue = 1.575 g (assumed copper).

- Mass of Zn dissolved = 2.500 − 1.575 = 0.925 g

- %Cu = (1.575 / 2.500) × 100 = 63.0 %

- %Zn = (0.925 / 2.500) × 100 = 37.0 %

Report the results to the appropriate number of significant figures.

8. Calculations — Density Method (Alternate Quantitative Approach)

If the alloy is a binary mixture and the densities of pure components are known, the volume fraction method can estimate composition.

Let alloy density = ρ_alloy, component densities ρ1 and ρ2, and mass fractions w1 and w2 (w1 + w2 = 1).

From rule of mixtures for density (approximate for homogeneous alloys):

1ρalloy=w1ρ1+w2ρ2

Solve for w1 and w2 given measured ρ_alloy. Convert mass fractions to percentages.

Example: For brass (Cu ρ = 8.96 g·cm⁻³, Zn ρ = 7.14 g·cm⁻³), if measured ρ_alloy = 8.40 g·cm⁻³, solve the equation to get approximate mass fractions.

9. Results and Discussion

- Tabulate the % composition for each alloy sample (from dissolution or density method).

- Compare calculated values with expected textbook compositions (e.g., typical brass ~60–70% Cu, 30–40% Zn). Discuss deviations and possible causes: impurities, incomplete dissolution, measurement errors, presence of other alloying elements.

- Discuss precision and accuracy: mention repeated trials, drying to constant mass, correct rinsing to avoid loss of residue, and calibration of balance.

10. Conclusion

Summarize findings: which constituents were identified in each sample, and the measured compositions (with uncertainties). State whether the hypothesis about alloy identity (brass, bronze, etc.) was confirmed by your tests.

11. Precautions and Safety

- Wear safety goggles, lab coat, and gloves at all times.

- Perform acid reactions under a fume hood or in a well-ventilated area.

- Use dilute acids (1–2 M) and avoid concentrated acids unless trained and equipped.

- Do not mix chemicals blindly; neutralize acid waste before disposal according to laboratory rules.

- Dry residues carefully — avoid overheating (which can oxidize metals or cause loss of mass).

- Use small sample sizes to limit hazards. Keep a spill kit and first aid available.

12. Bibliography / References

- B. R. Puri, L. R. Sharma, and M. S. Pathania — Principles of Physical Chemistry (selected lab methods)

- A. Textbook of Inorganic Chemistry (practical manual sections on metal analysis)

- Standard school/college chemistry lab manuals (qualitative inorganic analysis chapters)

- General safety and waste-disposal guidelines from your institution’s laboratory manual

13. Future Scope

- Use instrumental methods for precise and multi-element analysis: Atomic Absorption Spectroscopy (AAS), Inductively Coupled Plasma Optical Emission Spectroscopy (ICP-OES), X-Ray Fluorescence (XRF).

- Study microstructure with optical microscopy and SEM to correlate phases with composition.

- Perform hardness testing and correlate mechanical properties with composition.

- Explore corrosion rates of different alloy compositions and protective coatings.

14. Diagrams (suggested — draw or include images in final report)

- Figure 1: Density measurement setup (pycnometer/water displacement) — show sample suspended from balance into a beaker of water.

[Balance]

|

sample

|

(suspended)

------------- <-- water surface in beaker

| |

| Beaker |

|___________|